# PhotoFovea — Create Blog #4
First-Time Camera Setup Checklist

## Before You Begin (5 minutes)

Charge battery fully; keep a spare if available.

Insert and format storage card(s) in-camera.

Attach lens; remove both caps; confirm lens clicks/locks.

Set date/time and time zone (important for file organization).

Reset/initialize settings if using a used/borrowed camera (optional but helpful).

## Core Image Settings (Must-Do)

Set file format to RAW (NEF/CR3/ARW, etc.).

If available: choose 14-bit RAW (maximum tonal detail).

Set color space for previews/exports: sRGB is fine for beginners (RAW not affected).

Set Picture Control/Profile to Neutral (for honest in-camera previews).

## Exposure Defaults (Beginner-Safe)

Enable Auto ISO (recommended for beginners).

Set minimum shutter speed (e.g., 1/125 for general handheld).

Set maximum ISO (e.g., 6400 as a balanced starting point).

Set metering mode to Matrix/Evaluative.

## Autofocus Defaults (Predictable & Precise)

AF-S for stationary subjects; AF-C for moving subjects.

AF area mode: Single-Point AF for maximum control.

For action: Dynamic/Expanded AF area (9–25 points) if available.

Confirm focus priority vs release priority settings (prefer focus for beginners).

## White Balance & Image Review

Set White Balance to Auto (AWB 1 / Standard Auto).

Enable highlight warning (“blinkies”) if available.

Enable histogram display on playback.

Set playback zoom to 100% shortcut if possible.

## Dual Card Slot Strategy (If Available)

Choose Slot 2 role: Backup (recommended) or Overflow.

If backup: confirm RAW writes to both cards.

Label cards (C1, C2, etc.) and rotate after verified backups.

## Customize Buttons (Reduce Menu Diving)

Assign one button to ISO.

Assign one button to AF area mode or focus mode toggle.

Optionally assign a button to metering mode.

Save settings to a preset/bank once configured.

## First Test Shots (System Verification)

Shoot 5–10 RAW test frames in steady light.

Zoom to 100% to confirm critical focus.

Check histogram: avoid clipped highlights on important areas.

Confirm shutter speed is not too slow (no camera shake).

## Offload & Backup (Your Safety Net)

Copy RAW originals to Primary drive (NEF_Originals).

Import/convert to working DNGs on separate drive (DNG_Working) if that’s your workflow.

Back up both drives (local + offsite/cloud if available).

Only then format cards in-camera for the next shoot.