# PhotoFovea — Create Blog #6
Seeing Exposure: Histogram Field Checklist

## Before Shooting

Enable histogram display on image playback.

Enable highlight warning (“blinkies”).

Set image quality to RAW.

Set Picture Control/Profile to Neutral for honest previews.

## Initial Exposure Setup

Choose shooting mode (Aperture Priority or Manual + Auto ISO).

Set base ISO when light allows.

Select aperture based on depth-of-field intent.

Confirm minimum shutter speed is safe for focal length.

## Reading the Histogram

Check left edge for clipped shadows.

Check right edge for clipped highlights.

Identify where the majority of tones sit.

Remember: histogram shape reflects scene contrast, not correctness.

## Corrective Actions

If highlights clip: reduce exposure or protect highlights.

If shadows block: increase exposure or raise ISO.

Use exposure compensation when working in auto modes.

Bracket exposures for difficult contrast.

## Field Confidence Checks

Ignore LCD brightness; trust the histogram.

Zoom to 100% on critical areas.

Re-shoot immediately if tonal data is compromised.

## After the Shoot

Confirm exposure consistency across series.

Flag histogram-safe images for editing.

Back up RAW files before formatting cards.